| DD: Fuel Rail Pressure Temperature (FRPT) Sensor Introduction | DD: Pinpoint Tests |
| --- | --- |

WARNING: VEHICLE FUEL SYSTEMS ARE PRESSURIZED EVEN WHEN THE ENGINE IS NOT RUNNING. TO AVOID FIRE OR PERSONAL INJURY, DISABLE THE FUEL DELIVERY SYSTEM AND RELIEVE FUEL SYSTEM PRESSURE BEFORE REMOVING ANY FUEL SYSTEM COMPONENT. REFER TO THE FUEL SYSTEM INFORMATION AT THE BEGINNING OF PINPOINT HC. FAILURE TO FOLLOW THESE INSTRUCTIONS MAY RESULT IN PERSONAL INJURY.

Note: With the engine running, the FRP PID value may be 48-70 kPa (7-10 psi) higher than a fuel pressure reading taken with a mechanical gauge.

This pinpoint test is intended to diagnose the following:

- fuel rail pressure temperature (FRPT) sensor (9G756)

- harness circuits: FRP and FRT

- powertrain control module (PCM) (12A650)

### Tables and Graphs

FRPT SENSOR VOLTAGE AND PRESSURE SPECIFICATIONS

| Voltage | Pressure (kPa) | Pressure (psi) |
| --- | --- | --- |
| 4.5 | 482 | 70 |
| 3.9 | 413 | 60 |
| 3.4 | 344 | 50 |
| 2.8 | 275 | 40 |
| 2.2 | 207 | 30 |
| 1.6 | 138 | 20 |
| 1.1 | 69 | 10 |
| 0.5 | 0 | 0 |

FRPT SENSOR TEMPERATURE, VOLTAGE, AND RESISTANCE SPECIFICATIONS

| Temperature |  | Sensor |  |
| --- | --- | --- | --- |
| °C | °F | Volts | K Ohms |
| 100 | 212 | 0.47 | 2.073 |
| 95 | 203 | 0.54 | 2.405 |
| 90 | 194 | 0.61 | 2.800 |
| 85 | 185 | 0.70 | 3.273 |
| 80 | 176 | 0.80 | 3.840 |
| 75 | 167 | 0.92 | 4.524 |
| 70 | 158 | 1.06 | 5.351 |
| 65 | 149 | 1.21 | 6.356 |
| 60 | 140 | 1.38 | 7.584 |
| 55 | 131 | 1.56 | 9.091 |
| 50 | 122 | 1.77 | 10.949 |
| 45 | 113 | 1.99 | 13.252 |
| 40 | 104 | 2.23 | 16.123 |
| 35 | 95 | 2.48 | 19.720 |
| 30 | 86 | 2.74 | 24.253 |
| 25 | 77 | 3.00 | 30.000 |
| 20 | 68 | 3.26 | 37.332 |
| 15 | 59 | 3.50 | 46.745 |
| 10 | 50 | 3.73 | 58.911 |
| 5 | 41 | 3.95 | 74.745 |
| 0 | 32 | 4.13 | 95.501 |

### Fuel Rail Pressure Temperature (FRPT) Sensor Connector

| Pin | Circuit |
| --- | --- |
| 3 | FRT (Fuel Rail Temperature) |
| 1 | FRP (Fuel Rail Pressure) |
| 4 | SIGRTN (Signal Return) |
| 2 | VREF (Reference Voltage) |

### Powertrain Control Module (PCM) Connector

For PCM connector views or reference values, refer to Section 6 .

| Vehicle | Connector | Pin | Circuit |
| --- | --- | --- | --- |
| Escape/Mariner | 150 (50-50-50) Pin | E40 E28 E41 E37 | VREF FRT SIGRTN FRP |
| F-150, Mark LT | 190 Pin | E57 E19 E58 E32 | VREF FRT SIGRTN FRP |
| All other vehicles | 170 Pin | E57 E19 E58 E32 | VREF FRT SIGRTN FRP |

For DTC P0191, GO to DD11 .

#### DD11 DTC P0191: CHECK FOR FUEL PUMP DTCS

- Carry out the self-test.

Are DTCs P1233, P1234, P1235, P1236, P1237 or P1238 present?

| Yes | No |
| --- | --- |
| DISREGARD the current diagnostic trouble code (DTC) at this time. DIAGNOSE the next DTC. GO to Section 4, Diagnostic Trouble Code (DTC) Charts and Descriptions . | GO to DD12 . |

#### DD12 INSPECT ALL THE VACUUM HOSES CONNECTED TO THE INTAKE MANIFOLD FOR LEAKS

- Key in OFF position.

- FRPT Sensor connector connected.

- Key ON, engine running.

- Allow the engine idle to stabilize.

- Inspect all the vacuum hoses connected to the intake manifold for leaks.

Are any vacuum hose concerns present?

| Yes | No |
| --- | --- |
| ISOLATE the concern and REPAIR as necessary. CLEAR the DTCs. REPEAT the self-test. | GO to DD13 . |

#### DD13 CHECK THE FRPT CONNECTOR FOR DAMAGE OR CORROSION

- Key in OFF position.

- FRPT Sensor connector disconnected.

- Inspect the sensor, wiring, and connector for damage, corrosion, or water intrusion.

Is a concern present?

| Yes | No |
| --- | --- |
| REPAIR as necessary. CLEAR the DTCs. REPEAT the self-test. | GO to DD14 . |

#### DD14 CHECK THE FRP PID

Note: The fuel pressure is likely to increase after the fuel pressure is relieved with the system closed. The rate and amount of the fuel pressure increase is dependent upon the ambient air and fuel temperatures.

Note: Prepare to record the FRP PID value within 5 seconds after the engine is shut off and also after the fuel pressure is relieved.

- FRPT Sensor connector connected.

- Key ON, engine running.

- Allow the engine idle to stabilize.

- Access the PCM and monitor the FRP PID.

- Key in OFF position.

- Key ON, engine OFF.

- Record the FRP PID value within 5 seconds of the key off.

- Relieve the fuel pressure. Refer to the Workshop Manual Section 310-00, Fuel System for the Fuel System Pressure Release procedure.

- Disable the fuel pump.

- Key ON, engine OFF.

- Record the FRP PID value within 5 seconds of carrying out the fuel system pressure release procedure.

Is the difference between the recorded FRP PID values greater than 34 kPa (5 psi)?

| Yes | No |
| --- | --- |
| GO to Pinpoint Test HC . | GO to DD15 . |

#### DD15 COMPARE THE FRP PID TO THE MECHANICAL GAUGE

Note: Most mechanical gauges are referenced to atmospheric pressure. The FRPT sensor is referenced to manifold pressure. In order to make a valid comparison, the engine must be off.

Note: The vehicle may exhibit a long crank until the fuel system is pressurized.

- Key in OFF position.

- Connect a mechanical fuel pressure gauge.

- Key ON, engine OFF.

- Monitor the mechanical gauge.

- Access the PCM and monitor the FRP PID.

- Compare the FRP PID value to the mechanical gauge.

- Key in OFF position.

- Pressurize the fuel system. Refer to the Workshop Manual Section 310-00, Fuel System for the Fuel System Pressure Release procedure to pressurize the fuel system.

- Key ON, engine running.

- Allow the fuel pressure to stabilize.

- Key in OFF position.

- Key ON, engine OFF.

- Compare the FRP PID value to the mechanical gauge.

Are the FRP PID values within 34 kPa (5 psi) of the mechanical gauge readings?

| Yes | No |
| --- | --- |
| GO to DD28 . | INSTALL a new FRPT sensor. REFER to the fuel system WARNING information at the beginning of Pinpoint Test HC. GO to Pinpoint Test HC . REFER to the Workshop Manual Section 303-14, Electronic Engine Controls. CLEAR the DTCs. REPEAT the self-test. |

Note: DD28 suggest replacing the PCM