# Coolant Pump

Special Tool(s) / General Equipment

|  | 303-1556 Locking Tool, Timing Belt TKIT-2010B-FLM TKIT-2010B-ROW |
| --- | --- |
|  | 303-748 Locking Tool, Crankshaft TKIT-2010B-FLM TKIT-2010B-ROW |
| Hose Clamp Remover/Installer |  |
| 4 mm Drill Bit |  |

Removal

NOTE: The original coolant pump may not be equipped with a clutch and wiring harness connector.

- Refer to: Engine Cooling System Health and Safety Precautions (100-00 General Information, Description and Operation).

- Drain the cooling system.
Refer to: Engine Cooling System Draining, Vacuum Filling and Bleeding (303-03A Engine Cooling - 1.5L EcoBoost (118kW/160PS) – I4, General Procedures).

- Remove the engine appearance cover.

- Disconnect the electrical connector.

- Detach the wiring harness hose assembly and disconnect the electrical connector.

- Remove the following items:

  - Remove the air cleaner outlet pipe.
Refer to: Air Cleaner Outlet Pipe (303-12A Intake Air Distribution and Filtering - 1.5L EcoBoost (118kW/160PS) – I4, Removal and Installation).

- Remove the crankcase vent hose.
Use the General Equipment: Hose Clamp Remover/Installer

- Release the clamp and disconnect the bypass hose. Detach the wiring harness pin-type retainers and loosen the clamp.
Use the General Equipment: Hose Clamp Remover/Installer

- Remove the bolts and the TC inlet pipe.

- Loosen the coolant pump pulley bolts.

- Refer to: Jacking and Lifting - Overview (100-02 Jacking and Lifting, Description and Operation).

- Remove the nuts and the axle bearing support.

- NOTE: It may be necessary to rotate the halfshaft to remove the front lower bolt.

Remove the bolts and position the bracket support aside.

- Remove the engine block bolt.

- Install Special Service Tool: 303-748  Locking Tool, Crankshaft.

- NOTICE: Only rotate the crankshaft clockwise.

Rotate the crankshaft slowly until the crankshaft stops.

- Remove the following items:

  - Remove the generator.
Refer to: Generator - 1.5L EcoBoost (118kW/160PS) – I4 (414-02 Generator and Regulator, Removal and Installation).

  - Remove the engine mount.
Refer to: Engine Mount (303-01A Engine - 1.5L EcoBoost (118kW/160PS) – I4, Removal and Installation).

- Remove the bolts and the accessory drive belt tensioner.

  - Detach the wiring harness retainer and remove the nut.

  - Remove the bolt and the supporting bracket.

  - Remove the timing cover stud bolt.

  - Detach the cover from the retainer and open.

  - Remove the bolts and the coolant pump pulley.

  - Remove the engine mount stud.

  - Detach the wiring harness retainers.

  - Remove the bolt.

  - Detach the wiring harness retainer.

  - Remove the bolt.

  - Remove the bolts.

- NOTE: If the vehicle is equipped with a coolant pump that has a clutch, it may be necessary to break the timing belt cover during removal. Install a new timing belt cover.

Remove the timing belt cover.

- NOTE: Timing belt removal is only necessary if a new timing belt is being installed.

Install a new timing belt if the existing belt is contaminated by coolant or oil.
Refer to: Timing Belt (303-01A Engine - 1.5L EcoBoost (118kW/160PS) – I4, Removal and Installation).

- NOTE: The original coolant pump may not be equipped with a clutch and wiring harness connector.

Disconnect the electrical connector.

  - NOTICE: Only use the specified special tool.

NOTICE: Make sure that the crankshaft is not rotated while the special tool is installed.

Install Special Service Tool: 303-1556  Locking Tool, Timing Belt.

  - NOTICE: Note the position of the components before removal.

- WARNING: The tensioner is under spring tension. Be careful when handling the tensioner. Failure to follow this instruction may result in personal injury.

NOTICE: The crankshaft or camshafts must not be rotated while the timing belt is removed.

Rotate the timing belt tensioner and install a holding pin.
Install the General Equipment: 4 mm Drill Bit

- WARNING: The tensioner is under spring tension. Be careful when handling the tensioner. Failure to follow this instruction may result in personal injury.

Remove the bolt and timing belt tensioner.

  - Protect the timing belt and the lower timing belt cover from contamination by the coolant.

  - Remove the bolts and the coolant pump.

- Remove and discard the gasket.

Installation

NOTE: The replacement coolant pump is not equipped with a clutch and engine harness connector plug. Check OASIS for any applicable SSMs. If an SSM exists, follow SSM instructions. When the original coolant pump is equipped with a clutch and engine harness connector, cut the harness connector off, use heat shrink to seal the wire ends, and tape back the wires as needed.

- NOTE: Make sure that the mating faces are clean and free of foreign material.

Install a new gasket.

- Install the coolant pump and the bolts.
Torque:
  Stage 1:  53 lb.in (6 Nm)
  Stage 2:  89 lb.in (10 Nm)

- NOTE: Make sure that the components are installed to the position noted before removal.

- WARNING: The tensioner is under spring tension. Be careful when handling the tensioner. Failure to follow this instruction may result in personal injury.

  - Install the timing belt tensioner and tighten the bolt.
Torque: 18 lb.ft (25 Nm)

  - Remove the holding pin.
Remove the General Equipment: 4 mm Drill Bit

  - NOTE: Make sure that the components are installed to the position noted before removal.

  - Remove Special Service Tool: 303-1556  Locking Tool, Timing Belt.

- Remove Special Service Tool: 303-748  Locking Tool, Crankshaft.

- Install and tighten the engine block bolt.
Torque: 177 lb.in (20 Nm)

- Rinse residual engine coolant off the timing belt and the related drive components.

- Install the timing belt cover.

  - Install the timing belt cover bolts.
Torque: 89 lb.in (10 Nm)

  - Install the bolt.
Torque: 89 lb.in (10 Nm)

  - Install and tighten the engine mount stud.
Torque: 71 lb.in (8 Nm)

  - NOTE: Only tighten the bolts finger tight at this stage.

Install the coolant pump pulley and tighten the bolts.

  - Position the wiring aside assembly and attach the wiring harness retainer.

  - Install and tighten the bolt.

  - Attach the wiring harness retainers.

  - Close the cover and attach the retainer.

  - Install and tighten the timing cover stud bolt.
Torque: 89 lb.in (10 Nm)

  - Install the supporting bracket and tighten the nut.
Torque: 62 lb.in (7 Nm)

  - Install the bolt and attach the wiring harness retainer.
Torque: 17 lb.ft (23 Nm)

- Install the accessory drive tensioner and tighten the bolts.
Torque: 35 lb.ft (48 Nm)

- Install the following items:

  - Install the engine mount.
Refer to: Engine Mount (303-01A Engine - 1.5L EcoBoost (118kW/160PS) – I4, Removal and Installation).

  - Install the generator.
Refer to: Generator - 1.5L EcoBoost (118kW/160PS) – I4 (414-02 Generator and Regulator, Removal and Installation).

- Position the support bracket and tighten the bolts.
Torque: 35 lb.ft (48 Nm)

- NOTE: Make sure that a new component is installed.

Install the axle bearing support and tighten the nuts.
Torque:
  Stage 1:  44 lb.in (5 Nm)
  Stage 2:  18 lb.ft (25 Nm)
  Stage 3:  18 lb.ft (25 Nm)

- Install the underbody shield and the retainers.

- Tighten the bolts.
Torque: 18 lb.ft (24 Nm)

- Install the TC inlet pipe and tighten the bolts.
Torque: 71 lb.in (8 Nm)

  - Tighten the clamp.
Torque: 44 lb.in (5 Nm)

  - Connect the bypass hose and attach the wiring harness retainers.
Use the General Equipment: Hose Clamp Remover/Installer

- Install the vent hose.
Use the General Equipment: Hose Clamp Remover/Installer

- Install the following items:

  - Install the air cleaner outlet pipe.
Refer to: Air Cleaner Outlet Pipe (303-12A Intake Air Distribution and Filtering - 1.5L EcoBoost (118kW/160PS) – I4, Removal and Installation).

- Connect the electrical connector and attach the wiring harness retainer.

- Connect the electrical connector.

- Install the engine appearance cover.

- Fill the bleed the cooling system.
Refer to: Engine Cooling System Draining, Vacuum Filling and Bleeding (303-03A Engine Cooling - 1.5L EcoBoost (118kW/160PS) – I4, General Procedures).